CPE301 – SPRING 2021

# Design Assignment 4B

Student Name: Brian Wolak

Student #: 2000509437

Student Email: wolak@unlv.nevada.edu

Primary Github address: https://github.com/brianwolak/submission_da.git

Directory: submission_da/DA_4B at main · brianwolak/submission_da (github.com)

Task 1:

Task one for this design assignment involves writing a C code to interface a stepper motor and control its speed using a potentiometer connected and read from pin PCO.

Design Assignment 4B Task #1 Ports Used

Video Link:

https://youtu.be/VSgTz4x6sms

C Code:

#define F_CPU 16000000UL

#define BAUD 9600

#include <avr/io.h>

#include <util/setbaud.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <stdio.h>

volatile unsigned int pot;			//temporary read value from ADC

int delay = 0;				//OCR2A manipulation value

int loop = 0;					//timer looping value

char display1[20];				//display output string

//usart initialize

void USART_init(void)

{

UBRR0H = UBRRH_VALUE;

UBRR0L = UBRRL_VALUE;

UCSR0C = _BV(UCSZ01) | _BV(UCSZ00);

UCSR0B = _BV(RXEN0) | _BV(TXEN0);

}

//ADC initialize

void adc_init(void){

ADMUX = (0 << REFS1) | (1 << REFS0) | (0 << ADLAR) | (0 << MUX3) | (0 << MUX2) |

(0 << MUX1) | (0 << MUX0);

ADCSRA = (1 << ADEN) | (0 << ADSC) | (0 << ADATE) | (0 << ADIF) | (0 << ADIE) | (1

<< ADPS2) | (1 << ADPS1) | (1 << ADPS0);

}

//read ADC value

void read_adc(void){

ADCSRA |= (1 << ADSC);

while((ADCSRA & (1 << ADIF))==0);		//wait for ADIF flag

ADCSRA |= (1 << ADIF);			//reset ADIF flag

pot = ADC;					//read value

delay = (pot / 4);				//modify pot value for OCR2A

if(delay < 20){				//check min pot value

delay = 20;				//set min value if below 20

}

OCR2A = delay;				//set OCR2A to delay value

}

//timer setup

void timer_set(void){

TCCR2A |= (0 << COM2A1) | (0 << COM2A0) | (1 << WGM21) | (0 << WGM20);		//mode 2 CTC top 0xFF non-invert

TCCR2B |= (0 << WGM22) | (1 << CS22) | (1 << CS21) | (1 << CS20);			//1024 prescale set

}

void motor_delay(){

TCNT2 = 0;				//reset timer2

while (OCR2A != TCNT2);		//compare timer2 with OCR2A delay

loop++;				//increment loop

}

int main(void)

{

DDRB = 0x0F;				//set PORTD low bits to output

int position = 0;			//motor position variable

//initialize function call

USART_init();

//initialize function call

adc_init();

//initialize function call

timer_set();

//turn on interrupts

sei();

while (1)

{

read_adc();				//read pot value call

motor_delay();			//motor delay call

if(loop == 4){

switch(position){

case 0:					//motor case 0

PORTB = 0x03;		//pin 0 and 1 high

position++;			//increment position

loop = 0;			//reset loop

break;

case 1:					//motor case 1

PORTB = 0x06;		//pin 1 and 2 high

position++;			//increment position

loop = 0;			//reset loop

break;

case 2:					//motor case 2

PORTB = 0x0C;		//pin 3 and 4 high

position++;			//increment position

loop = 0;			//reset loop

break;

case 3:					//motor case 3

PORTB = 0x09;		//pin 0 and 3 high

position = 0;		//reset position

loop = 0;			//reset loop

break;

}

}

}

}

Design Assignment 4B Task 1 Board Setup

Design Assignment 4B Task #1 Circuit

Task 2:

Task two for this design assignment involves writing a C code to interface a servo motor and control its position using a potentiometer connected and read from pin PCO. The servo motor will perform a smooth 1800 sweep when a sweep of the potentiometer is read with potentiometer value of zero corresponding to 00 and 5V to the 1800 position.

Design Assignment 4B Task #2 Ports Used

Video Link:

https://youtu.be/VaR87Lpuei0

C Code:

#define F_CPU 16000000UL		/* Define CPU Frequency 16MHz */

#include <avr/io.h>			/* Include AVR std. library file */

#include <util/delay.h>		/* Include Delay header file */

volatile unsigned int pot;		//temporary read value from ADC

//ADC initialize, PC0 input

void adc_init(void){

ADMUX = (0 << REFS1) | (1 << REFS0) | (0 << ADLAR) | (0 << MUX3) | (0 << MUX2) |

(0 << MUX1) | (0 << MUX0);

ADCSRA = (1 << ADEN) | (0 << ADSC) | (0 << ADATE) | (0 << ADIF) | (0 << ADIE) | (1

<< ADPS2) | (1 << ADPS1) | (1 << ADPS0);

}

//read ADC value

void read_adc(void){

ADCSRA |= (1 << ADSC);

while((ADCSRA & (1 << ADIF))==0);		//wait for ADIF flag

ADCSRA |= (1 << ADIF);			//reset ADIF flag

pot = ADC;					//read value

OCR1A = 50 + pot / 2;			//set pot value to OCR1A with

}

//timer1 setup function

void t1_setup(void){

TCCR1A|=(1<<COM1A1)|(1<<COM1B1)|(1<<WGM11);        //NON Inverted PWM

TCCR1B|=(1<<WGM13)|(1<<WGM12)|(1<<CS11)|(1<<CS10); //PRESCALER=64 MODE 14(FAST PWM)

ICR1=4999;					//fPWM=50Hz (Period = 20ms Standard)

}

int main(void)

{

t1_setup();			//timer1 intialize

adc_init();			//initialize ADC

DDRB|=(1<<PINB1);   		//PWM output on PB1

while(1)

{

read_adc();		//read ADC

}

}

Design Assignment 4B Task 2 Board Setup

Design Assignment 4B Task #2 Circuit