CPE301 – SPRING 2021

# Midterm II

Student Name: Brian Wolak

Student #: 2000509437

Student Email: wolak@unlv.nevada.edu

Primary Github address: https://github.com/brianwolak/submission_da.git

Directory: submission_da/MIDTERM_2 at main · brianwolak/submission_da (github.com)

Task 1:

In task one a C code is written to program the ATmega328pb to drive a DC motor in clockwise for 5 seconds and then counterclockwise for 5 seconds using a giving PWM. The code should repeat forever switching from clockwise to counterclockwise. For this experiment my geared motor has burned out and I was forced to use a regular DC motor in its place.

ATmega328pb Ports Used in Task 1

Video Link:

https://youtu.be/uggxgGmAdAE

C Code:

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <stdio.h>

int direction = 0;

volatile int count = 0;

//timer setup

void timer_set(void){

//motor PWM timer

TCCR2A |= (0 << COM2A1) | (1 << COM2A0) | (1 << WGM21) | (1 << WGM20);		//mode 7, OC2A toggle

TCCR2B |= (1 << WGM22) | (1 << CS22) | (1 << CS21) | (1 << CS20);			//1024 prescale set

OCR2A = 0x10;

//delay timer

TCCR1A |= (0 << COM1A1) | (0 << COM1A0) | (0 << COM1B1) | (0 << COM1B0) | (0 << WGM11) | (0 << WGM10);

TCCR1B |= (0 << WGM13) | (1 << WGM12) | (1 << CS12) | (0 << CS11) | (1 << CS10);

TIMSK1 |=  (1 << OCIE1A);

OCR1A = 0x3D07;

}

int main(void)

{

DDRD |= (1<<2) | (1<<3);		//set PORTD output bits 2 & 3

DDRB |= (1<<3);			//set PORTB output bit 3 (OC2A)

//initialize function call

timer_set();

//turn on interrupts

sei();

while (1)

{

if (direction == 0){

PORTD &= ~(1<<2);			//PINC2 low

PORTD |= (1<<3);			//PINC3 high

}

if (direction == 1){

PORTD |= (1<<2);			//PINC2 high

PORTD &= ~(1<<3);			//PINC3 low

}

}

}

ISR(TIMER1_COMPA_vect){

count++;			//increment 5s timer

if (count == 5){		//check if 5s timer

direction ^= 1;		//flip direction

count = 0;			//reset timer

}

}

Midterm II Task 1 Pulseview showing 5s swap from CW to CCW rotation

Midterm II Circuit – Task 1

Midterm II Circuit – Task 1

Task 2:

In task two, a potentiometer value (0~1023) is read using ADC from PC0 and translated to a (0~255) PWM duty cycle to control the speed of a DC motor. This task was achieved using Timer 2 as requested by the midterm document.

ATmega328pb Ports Used in Task 2

Video Link:

https://youtu.be/BOGM-Dba--4

C Code:

#define F_CPU 16000000UL

#define BAUD 9600

#include <avr/io.h>

#include <util/setbaud.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <stdio.h>

volatile unsigned int pot;			//temp read value from ADC

//ADC initialize PC0 input

void adc_init(void){

ADMUX = (0 << REFS1) | (1 << REFS0) | (0 << ADLAR) | (0 << MUX3) | (0 << MUX2) |

(0 << MUX1) | (0 << MUX0);

ADCSRA = (1 << ADEN) | (0 << ADSC) | (0 << ADATE) | (0 << ADIF) | (0 << ADIE) | (1

<< ADPS2) | (1 << ADPS1) | (1 << ADPS0);

}

//read ADC value

void read_adc(void){

ADCSRA |= (1 << ADSC);

while((ADCSRA & (1 << ADIF))==0);		//wait for ADIF flag

ADCSRA |= (1 << ADIF);			//reset ADIF flag

pot = ADC;					//read value

}

//timer setup

void timer_set(void){

TCCR2A |= (1 << COM2A1) | (1 << WGM21) | (1 << WGM20);	//mode 3 fast PWM top 0xFF non-invert

TCCR2B |= (1 << CS22) | (1 << CS21) | (1 << CS20);	//1024 prescale set

}

int main(void)

{

DDRB |= (1<<3);				//set PORT B output PIN3

DDRD |= (1<<0) | (1<<1);			//set PORTD outputs

//initialize function call

adc_init();

//initialize function call

timer_set();

PORTD |= (1<<0);			//PINC0 high

//PORTD &= ~(1<<1);			//PINC0 low

while (1)

{

read_adc();			//read ADC

OCR2A = pot /4;		//set OCR2A to pot2 value

}

}

Midterm II Task 2 Pulseview showing a duty cycle sweep using the potentiometer

Midterm II Circuit – Task 2

Midterm II Circuit – Task 2

Task 3:

In task three the CCP capture pin is used to determine the speed of a DC motor using 1x mode. The motor speed is controlled using a 10k potentiometer connected to PINC 0 and read by ADC with the capture being read through PINB 0. Below you can see the output from the terminal as well as C code verifying functionality.

ATmega328pb Ports Used in Task 3

Video Link:

https://youtu.be/gCH52aHjdlk

C Code:

#define F_CPU 16000000UL

#define BAUD 9600

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <util/setbaud.h>

#include <stdlib.h>

#include <stdio.h>

volatile uint16_t pot;		//global pot variable

volatile float rpm, rps;		//rev per min, rev per sec

volatile uint32_t tick_rev;		//ticks per rev

volatile uint32_t rev_count;	//revolution count

volatile uint16_t TIMER1ovf;	//overflow value

volatile uint32_t tick1;		//temp tick value

volatile uint32_t tick_tot;		//total ticks

char display[20];			//print variable to terminal

//ADC initialze using PC0 input

void adc_init(void){

ADMUX = (0<<REFS1) | (1<<REFS0) | (0<<ADLAR) | (0<<MUX3) | (0<<MUX2) | (0 << MUX1) | (0 << MUX0);

ADCSRA = (1<<ADEN) | (0<<ADSC) | (0<<ADATE) | (0<<ADIF) | (0<<ADIE) | (1<<ADPS2) | (1<<ADPS1) | (1<<ADPS0);

}

//ADC read

void read_adc(void){

ADCSRA |= (1<<ADSC);

while((ADCSRA & (1<<ADIF))==0);		//wait for flag

ADCSRA |= (1<<ADIF);				//clear flag

pot = ADC;					//read ADC

OCR1A = pot;					//set OCR1A

}

//USART initialize

void USART_init(void){

UBRR0H = UBRRH_VALUE;

UBRR0L = UBRRL_VALUE;

UCSR0C = _BV(UCSZ01) | _BV(UCSZ00);

UCSR0B = _BV(RXEN0) | _BV(TXEN0);

}

//print to terminal function

void USART_tx_string(char *data){

while((*data != '\0')){			//while data is not empty

while(!(UCSR0A & (1<<UDRE0)));	//while data register is not empty

UDR0 = *data;				//UDR0 register gets data

data++;				//next data value

}

}

//timer setup

void timer_setup(void){

TCCR1A |= (1<<COM1A1) | (1<<WGM11) | (1<<WGM10);		//mode 7 fast PWM

TCCR1B |= (1<<ICES1) | (1<<WGM12) | (1<<CS10);		//prescale of 1

TIMSK1 |= (1<<ICIE1) | (1<<TOIE1);				//set bits for interrupts

}

//RPM calculate

void rpm_calc(void){

rps = 15625.0 / tick_tot;		//calc rpm

rpm = rps * 60;			//calc rps

if(rpm < 5)				//check if RPM out of range

rpm = 0;				//set RPM out of range

}

int main(void){

DDRB |= (1<<1);			//output set

PORTB |= (1<<0);			//pull up set

//setup functions

timer_setup();

adc_init();

USART_init();

sei();

while (1){

read_adc();				//read ADC and set OCR1A

rpm_calc();				//calulate rpm

USART_tx_string("Motor RPM = ");

//display rpm to terminal

snprintf(display, sizeof(display), "%.2f ", (float)rpm);

USART_tx_string(display);

USART_tx_string(" \r\n");

}

}//TIMER1 capture ISR

ISR(TIMER1_CAPT_vect) {

tick1 = ICR1;			//record tick value

tick_tot = (uint32_t)(tick1) + ((uint32_t)TIMER1ovf * 0x0400L);

rev_count++;			//increment revolution count

TCNT1 = 0;			//reset TIMER1

TIMER1ovf = 0;		//reset overflow

}

//TIMER1 overflow

ISR(TIMER1_OVF_vect) {

TIMER1ovf++;			//increment overflow

}

TASK 3 Terminal output displaying motor RPM

Midterm II Circuit – Task 3

Midterm II Circuit – Task 3

Task 4:

In task four the PID algorithm is used to control a DC motor speed, and maintain the motor’s speed adjusting for correction when needed. The motor PWM is supplied using OC1A as the output on PINB 1 and the capture is read through PINB0. Below is the C Code and SERIALPLOT outputs along with terminal output displaying the proper functionality.

ATmega328pb Ports Used in Task 4

Video Link:

https://youtu.be/MDKxZyZVY60

C Code:

#define F_CPU 16000000UL	//clock speed

#define BAUD 9600			//baud rate

#define IMEM 20

#define Pvalue 1.2

#define Ivalue 1.2

#define Dvalue 1.2

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/setbaud.h>

#include <stdlib.h>

#include <stdio.h>

volatile uint16_t pot;				//global ADC variable

volatile uint32_t rev_count, tick1, tick2, cap1,

cap2, cap3, cap4, avg1, avg2, tick_tot;

int count = 0, edge = 0, finish = 0, tick_count = 0;

volatile int16_t duty = 0, duty_max = 250;

volatile int32_t mean_I_err[IMEM], PID = 0, err_new = 0, err_old = 0,

mean_I_error = 0.0, I_err = 0.0,

P_err = 0, D_err = 0;

volatile uint16_t TIMER1ovf;			//overflow value

volatile float rpm, rps;

volatile double SETSPEED = 60, CURSPEED = 0;

char out[20], out1[20], out2[20];			//terminal print variables

//ADC initialize

void adc_init(void){

ADMUX = (0<<REFS1) | (1<<REFS0) | (0<<ADLAR) | (0<<MUX3) | (0<<MUX2) | (0 << MUX1) | (0 << MUX0);

ADCSRA = (1<<ADEN) | (0<<ADSC) | (0<<ADATE) | (0<<ADIF) | (0<<ADIE) | (1<<ADPS2) | (1<<ADPS1) | (1<<ADPS0);

}

//read the ADC value

void read_adc(void){

ADCSRA |= (1<<ADSC);

while((ADCSRA & (1<<ADIF))==0);		//wait for flag

ADCSRA |= (1<<ADIF);				//clear flag

pot = ADC;					//read ADC

}

//initialize USART

void USART_init(void)

{

UBRR0H = UBRRH_VALUE;

UBRR0L = UBRRL_VALUE;

UCSR0C = _BV(UCSZ01) | _BV(UCSZ00);

UCSR0B = _BV(RXEN0) | _BV(TXEN0);

}

//print to terminal function

void USART_tx_string(char *data){

while((*data != '\0')){			//while data is not empty

while(!(UCSR0A & (1<<UDRE0)));	//while data register is not empty

UDR0 = *data;				//UDR0 register gets data

data++;				//next data value

}

}

//timer setup function

void timer_setup(void){

//compare timer

TCCR2A |= (1<<WGM21) | (1<<WGM20);

TCCR2B |= (1<<CS22) | (1<<CS21) | ( 1<<CS20);

TIMSK2 |= (1<<OCIE2A);

//dc motor and capture timer

TCCR1A |= (1<<COM1A1) | (1<<WGM11) | (1<<WGM10);	//mode 7 fast PWM

TCCR1B |= (1<<ICES1) | (1<<WGM12) | (1<<CS10);	//prescale of 1

TIMSK1 |= (1<<ICIE1) | (1<<TOIE1);			//interrupt set

}

int main(void)

{

DDRB |= (1<<1);	//set PORTB outputs

PORTB |= (1<<0);	//set pull up resistor

//setup functions

timer_setup();

adc_init();

USART_init();

sei();

//initialize array

for(int i = 0; i < IMEM-1; i++){

mean_I_err[i] = (int32_t)0.0;

}

while (1)

{

rps = 15625.0 / tick_tot;	//calc revolutions per second

rpm = rps*60;			//calc revolutions per minute

CURSPEED = rpm;		//set RPM to curretn speed

snprintf(out, sizeof(out), "%.2f ", (float)rpm);	//print speed

USART_tx_string(out);

USART_tx_string(" \r\n");

}

}

//PID algorithm

ISR(TIMER2_COMPA_vect){

err_old = err_new;			//save new error

err_new = SETSPEED - CURSPEED;	//calc speed error

P_err = err_new;			//save new to last

mean_I_err[IMEM-1] = (int32_t)err_new;

mean_I_error = 0;

//shift left and add all

for(int i=0; i<IMEM-1; i++){

mean_I_err[i] = (int32_t)mean_I_err[i+1];

mean_I_error += (int32_t)mean_I_err[i];

}

I_err = (int32_t)mean_I_error;

D_err = (int32_t)err_new - err_old;

//calc PID

PID = (int32_t)Pvalue * P_err + Ivalue * I_err * .1 + Dvalue * D_err / .1;

//PID out of bounds check

if(PID > duty_max)

PID = duty_max;

else if(PID < 0)

PID = 0;

duty = (uint16_t)PID;

OCR1A = duty * 7.5;		//OCR1A scale calculation

}

//PID CAPTURE ISR

ISR(TIMER1_CAPT_vect) {

//rising edge 1

if(count==0 && edge==0){

tick1 = ICR1;			//save tick time stamp

cap1 = (uint32_t)(tick1) + ((uint32_t)TIMER1ovf*0x0400L);

rev_count++;			//increment revolution counter

count++;			//increment count

edge = 1;			//toggle edge value

TCCR1B ^= (1 << ICES1); 	//toggle capture select

}

//falling edge 1

else if(count==1 && edge==1){

tick1 = ICR1;			//save tick time stamp

cap2 = (uint32_t)(tick1) + ((uint32_t)TIMER1ovf*0x0400L);

rev_count++;			//increment revolution counter

count++;			//increment count

edge = 0;			//toggle edge value

TCCR1B ^= (1 << ICES1); 	//toggle capture select

}

//rising edge 2

else if(count==2 && edge==0){

tick1 = ICR1;			//save tick time stamp

cap3 = (uint32_t)(tick1) + ((uint32_t)TIMER1ovf*0x0400L);

rev_count++;			//increment revolution counter

count++;			//increment count

edge = 1;			//toggle edge value

TCCR1B ^= (1 << ICES1);	//toggle capture select

}

//falling edge 2

else if(count==3 && edge==1){

tick1 = ICR1;			//save tick time stamp

cap4 = (uint32_t)(tick1) + ((uint32_t)TIMER1ovf*0x0400L);

rev_count++;			//increment revolution counter

count = 0;			//reset count

edge = 0;			//toggle edge value

TCCR1B ^= (1 << ICES1); 	//toggle capture select

finish = 1;			//set final position value

}

//calculate avg / restart

if(finish == 1){

avg1 = cap3 - cap1;		//avg1 calculation

avg2 = cap4 - cap2;		//avg2 caclulation

tick_tot = (avg1 + avg2) / 2;	//total ticks

TCNT1 = 0;				//reset timer

TIMER1ovf = 0;			//reset overflow

finish = 0;				//reset final position value

}

}

//TIMER1 OVF ISR

ISR(TIMER1_OVF_vect) {

TIMER1ovf++;			//increment overflow count

}

Midterm II Circuit - Task 4

Midterm II Circuit – Task 4

TASK #4 SerialPlot Output – showing PID maintain 60 rpm

TASK #4 Terminal Output showing PID control maintain 60 rpm

Github Link:

submission_da/MIDTERM_2 at main · brianwolak/submission_da (github.com)

Video Links:

Task1:  https://youtu.be/uggxgGmAdAE

Task2: https://youtu.be/BOGM-Dba--4

Task3: https://youtu.be/gCH52aHjdlk

Task4: https://youtu.be/MDKxZyZVY60